|  | Industrial Waste Control Group 9300 W. Jefferson Avenue, Suite 210 Detroit, Michigan 48209 |
| --- | --- |

GUIDELINES FOR PREPARATION - SLUG CONTROL / SPILL PREVENTION PLAN

LEGAL AUTHORITY

The General Pretreatment Regulation 40 CFR 403,8(f)(2)(v) provides authority to the Publicly Owned Treatment Works (POTW), in this case the Water Resource Recovery Facility­ Great Lakes Water Authority (GLWA), to request any Industrial User (IU) to develop a Slug Control / Spill Prevention Plan in order to order to protect the sewer from slug and/or accidental discharges.

The GLWA Rules, Section II-303 declares that each IU discharging into the GLWA sewerage collection and treatment system shall provide protection from the accidental discharges of prohibited materials and other substances. Further, the Rules add that facilities and measures to prevent and abate accidental discharges should be provided and maintained at the owner's cost or expenses.

DEFINITIONS

SLUG DISCHARGE is any discharge of a non-routine, episodic nature including but not limited to an accidental spill or a non-customary batch discharge at a flow rate and/or concentration, which will cause interference or pass through at POTW.

UPSET means an exceptional incident in which there is unintentional and temporary noncompliance with the categorical pretreatment standards, or local pretreatment standards, because of factors beyond the reasonable control of the IU.  An upset does not include noncompliance to the extent caused by operational error, improperly designed treatment facilities, inadequate treatment facilities, lack of preventive maintenance, or careless or improper operation.

An upset shall constitute an affirmative defense to an action brought for noncompliance with limits imposed under the permit if the following requirements are met:

- An IU who wishes to establish the affirmative defense shall demonstrate, through properly signed, contemporaneous operating logs, or other relevant evidence that:

- An upset occurred and IU can identify the specific cause(s) of the upset,

- The facility was at the time being operated in a prudent and workman-like manner and in compliance with applicable operation and maintenance procedures,

- The IU has submitted the information on the upset to the Authority, orally or in writing, within twenty-four (24) hours of becoming aware of the upset (if this information is provided orally, a written submission must be made within five (5) calendar days).

- In any enforcement proceeding, the IU seeking to establish the occurrence of an upset shall have  the burden of proof.

- The IU shall control production of all discharges to the extent necessary to maintain compliance with this rule upon reduction, loss, or failure of its treatment facility until the facility is restored or an alternative method of treatment is provided. This requirement applies in the situation where, among other things, the primary source of power of the treatment facility is reduced, lost, or fails.

BYPASS means the intentional diversion of waste streams from any portion of an IU’s treatment facility.  Bypass is prohibited, and the Authority may take enforcement action against an IU for a bypass, unless:

- Bypass was unavoidable to prevent loss of life, personal injury, or severe property damage,

- There were no feasible alternatives to the bypass, such as the use of auxiliary treatment facilities, retention of untreated wastes, or maintenance during normal periods of equipment downtime.  This condition is not satisfied if adequate backup equipment should have been installed in the exercise of reasonable engineering judgment to prevent a bypass which occurred during normal periods of equipment downtime or preventative maintenance; and

3.	The IU submitted notices as required.

MATERIALS PROHIBITED FROM REACHING THE SEWER

GENERAL PROHIBITIONS

An IU shall not introduce into a POTW any pollutant(s) which cause pass through or   interference.

A list of EPA priority pollutants and Michigan critical materials is attached for your reference/use.

SPECIFIC PROHIBITIONS

- Pollutants which create a fire or explosion hazard in the POTW,

- Pollutants which will cause corrosive structural damage to the POTW,

- Solid or viscous pollutants in amounts which will cause obstruction to the flow in the POTW result,

- Any pollutant, including Biochemical Oxygen Demand (BOD) released in a discharge at a flow rate and/or pollutant concentration which will cause interference with the POTW,

- Heat in amounts which will inhibit biological activity in the POTW resulting in interference,

- Petroleum oil, non-biodegradable cutting oil, or products of mineral oil origin in amounts that will cause interference or pass through,

- Pollutants which result in the presence of toxic gases, vapors, or fumes within the POTW in a quantity that may cause acute worker health and safety problems,

- Any trucked or hauled pollutants, except at discharge points designated by the POTW.

CONTENTS OF THE PLAN

- Facility Identification

- Process description and schematic of wastewater flow

- Inventory of materials stored, raw materials, liquid wastes, by-products/products, solvents, and/or oils

- Procedures for slug control / spill prevention and response

- Method of disposal

- Notification requirements

- Slug history

- Inspection procedure

- Employee training

- Implementation date

- Certification of the plan

The nature and volume of information to be provided under each section as described in the subsequent ‘Guidelines for Preparation’ followed by a blank form for your use and/or reference.

GUIDELINES FOR PREPARATION

- Facility Identification

This section contains general information and is self-explanatory

- Process Description and Schematic of Wastewater Flow

This section should contain the following information related to the facility identified in Section I above.

- Product or Service Operation details,

- Description of discharges including non-routine batch discharges,

- Number of shifts per day, shift hours, the number of employees in each shift, and number of working days per week,

- A drawing of appropriate size showing the following details; facility layout, property boundaries, entrance and exit routes to facility, areas occupied by manufacturing service or commercial activities, storage area of hazardous or prohibited or listed materials, solvents- and oils, loading and unloading facilities, waste or wastewater handling, storage and treatment facilities, direction of drainage from storage areas, waste handling, process storage and treatment areas, floor drains, pipes and channels which lead away from potential spill areas identifying where the drains connect to sanitary sewer/holding tank, flow directions with flow rates, tank capacities, treatment system, GLWA and company sampling points and outlet connection to the street sewer.

- Inventory of Materials Stored in the Facility

A list of hazardous / listed / prohibited materials, by-products or products, liquid wastes, solvents and oils or any other materials such as large quantities of non­contact cooling water and/or wastewater containing non-toxic organics, solids, and nutrients stored inside the facility and their approximate average and maximum quantities with concentrations stored in the facility should be provided.  Also, show their respective locations on the drawing explained in Section II above. Only materials which are in a form which could readily be carried into the wastewater treatment plant and which constitute a concentration of 5% or greater on a dry weight basis in the raw materials, chemical solutions or waste material are required to be reported.

- Procedures for Slug Control / Spill Prevention and Response

Details of measures and installations required to protect the sewer system from slug/spill discharges should be explained.

  - Equipment

Identify the type of equipment and structure used in slug prevention and response activities and their location in the facility (dikes, berms, sealed drains, alarms, leak detection equipment, diversionary structures, sumps, protective equipment, decontamination equipment, ventilation equipment, absorbents, etc.)

  - Procedures

Provide procedures to prevent adverse impact on the GLWA sewer system from slug discharges, including handling and transferring of materials, loading and unloading operation, a control of plant site run-off, etc.

- Method of Disposal

Briefly describe the procedures for disposing of or treating spilled materials in your facility.

- Notification and Report Requirements

This section should contain a list of the federal, state, local agencies, contractors, consultants, etc., who will be informed during slug discharge. The following is the procedure required for notifying the GLWA on slug incidents:

Notification to GLWA

Slug Loading/Accidental Discharge

Within one (1) hour of becoming aware of a discharge entering into the sewer, the company shall telephone the GLWA System Control Center, within twenty-four (24) hour phone number at (313) 267-6000 and inform the Authority about the details of the discharge.

Upset at the IU's Pretreatment Facility

Within twenty-four (24) hours of becoming aware of an upset, the company shall telephone the GLWA at the System Control Center (313) 267-6000 and inform the Authority about the details of the upset and discharge,

Unanticipated Bypass of Waste Pretreatment Facility

Within twenty-four (24) hours of becoming aware of the bypass, the company shall telephone the GLWA at the System Control Center (313) 267-6000 inform the Authority about the details of the discharge,

All these notifications shall include the name of the caller, location, and time of discharge, type of wastewater, estimated concentration and volume.

Submission of Report

For the above mentioned three (3) incidents, a written report shall be submitted within five (5) calendar days of becoming aware of the incident This report shall contain the following:

- A description of the discharge and the cause of the incident,

- The duration of the incident including exact dates and times or, if not corrected, the anticipated time the incident is expected to continue,

- Steps being taken and/or planned to reduce, eliminate. and prevent similar future occurrences of the incident.

Anticipated Bypass

If an IU anticipates the need for a bypass, it shall submit a prior notice to the Authority, if possible at least ten (10) days before the date of the bypass.  The report shall be accompanied by the analytical data, which shows the characteristics of the material to be bypassed. Upon evaluation, the Authority provides the IU with  its determination on the bypass.

| Notification and Reporting Requirements for Specific Incidents |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| INCIDENT | DEFINITION | NOTIFICATION | REPORT | CONTACT | TELEPHONE NO. |
| Spill 40 CFR 403.12f | Accidental discharge to the city sewer system | Shall call within (1) hour of becoming aware of the spill | Shall submit within (5) days of the spill | William Ware System Control | (313) 297-5857 (313) 267-6000 (24 hour number) |
| Upset 40 CFR 403.16 | Pretreatment system upset causing noncompliance | Shall call within (24) hours of becoming aware of the upset | Shall submit within (5) days of the spill | IWC Inspector System Control | (313) 297-5850 (313) 267-6000 (24 hour number) |
| Bypass 40 CFR 403.17 | Intentional diversion of waste streams from the pretreatment facility - Anticipated Bypass - Unanticipated Bypass | Shall call within (24) hours of becoming aware of the bypass | - Shall submit sample reports of material to be bypassed, 10 days prior to the bypass - Shall submit report within (5) days of the bypass | System Control | (313) 267-6000 (24 hour number) |
| Noncompliance 40 CFR 403.12g | Violation of any regulated parameters of wastewater to the sewer, identified while self-monitoring | Shall call within (24) hours of becoming aware of the violation | Shall submit within (30) days of the violation | IWC Inspector System Control | (313) 297-5850 (313) 267-6000 (24 hour number) |
| Days denotes calendar days |  |  |  |  |  |

| Details of the Reports for Specific Incidents |  |  |
| --- | --- | --- |
| SPILL | Causes and details of the discharge, and measures to be taken to prevent similar future occurrences. |  |
| UPSET | A description of the discharge and cause of noncompliance; the period of noncompliance including exact dates and times or, if not corrected, the anticipated time the noncompliance is expected to continue; steps being taken and/or planned to reduce, eliminate and prevent recurrence of the noncompliance. |  |
| BYPASS | i) | Anticipated - prior notice, with the sample analysis of the material that is going to be bypassed, for the Authority’s approval to bypass. |
|  | ii) | Unanticipated - description of the bypass and its cause; the duration of the bypass, including exact dates and times, and, if the bypass has not been corrected, the anticipated time it is expected to continue, and steps taken or planned to reduce, eliminate and prevent recurrence of the bypass. |
| NONCOMPLIANCE | The user shall repeat the sampling and analysis and submit the results of the repeat analysis. |  |

All reports shall be submitted to the following address:

Great Lakes Water Authority

Industrial Waste Control Group

9300 W. Jefferson, Ste. 210

Detroit, Michigan 48209-2676

A notice shall be permanently posted on the company's bulletin board or ether prominent place(s) advising employees to contact GLWA System Control   Center (24 hr. phone no.) in the event of a potential excessive discharge to the sewer system.

- Company Slug History

This section should contain the details of the slug discharges that occurred in the facility during the past three (3) years.  It should also contain the dates, and volumes of wastewater/chemicals discharged into the sewer, names of pollutants and their respective concentrations, authorities informed and remedial measures instituted.

- Inspection Procedure

This section should contain details of inspection schedules established to check the conditions of the IU's facilities, equipment, tools, supplies, etc., to detect and correct the potential sources of slug discharges and related containment activities. It should also contain the skill level of the person, performing inspection periodicity of inspection, areas covered, records maintained, and the follow up procedures of inspection report.

- Employee Training

This section should contain the skill types of the IU's employees and the number of employees who were formally instructed or trained periodically to act during the incidents of slug discharges. List the formal classroom lectures and demonstrations the operators attended. Also list the ongoing training classes conducted periodically for this purpose.

- Implementation Date

This information is required to determine the time period from when the protection against slug discharges has been provided. List the date, month and the year on which the Slug Control / Spill Prevention Plan being submitted to the GLWA took effect.

- Certification of Implementation

The Slug Control/Spill Prevention Plan submitted by your company shall be signed by an authorized representative.  The Certification Statement shall read as follows:

I certify that the information provided in this Slug Control / Spill Prevention Plan is to the best of·my knowledge, accurate and true, and that the incidental slug control / spill prevention measures described in this plan will be implemented as described.

Print the name of the certifying officer, title, and the date of certification.

This Slug Control / Spill Prevention Plan shall be reviewed and immediately amended, if necessary, whenever:

- The facility permit is reviewed (only the IU with Wastewater Discharge Permit).

- The plan fails in an emergency.

- The facility changes in its design, construction, operation1 maintenance, or other circumstances in a way that materially increases the potential for slug / accidental discharges to the sewer.

- The list of emergency coordinators or facility contacts persons change.

INDUSTRIAL  USER    SLUG CONTROL / SPILL  PREVENTION  PLAN

| - Facility Notification |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Facility Name |  | **** |  |  |  |  |
|  | Facility Address |  | **** |  |  |  |  |
|  | Mailing Address |  | **** |  |  |  |  |
|  | Facility Contact Person |  | **** |  |  |  |  |
|  | Title | **** |  | Work Phone No. / Cell No. | **** | **** |  |
|  | E-Mail Address |  | **** |  |  |  |  |
|  | Facility Spill Coordinator |  | **** |  |  |  |  |
|  | Title | **** |  | Work Phone No. / Cell No. | **** |  | **** |
|  | E-Mail Address |  | **** |  |  |  |  |
|  | Other Emergency Coordinator |  | **** |  |  |  |  |
|  | Title | **** |  | Work Phone No. / Cell No. | **** |  | **** |
|  | E-Mail Address |  | **** |  |  |  |  |

| - Process Description and Schematics |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Type of Business / Manufacturer |  |  |  |  |  |  |
|  | **** |  |  |  |  |  |  |
|  | - Description of Discharges (including non-routine batch discharges) |  |  |  |  |  |  |
|  | **** |  |  |  |  |  |  |
|  | - Operating Schedule | **** |  |  |  |  |  |
|  | Number of Employees | 1st Shift | **** | 2nd Shift | **** | 3rd Shift | **** |

|  | - Attach Schematics or Drawings |
| --- | --- |

| - Inventory of Materials Stored in the Facility |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Material Name* | Quantity** gallons (otherwise specify) | Location in the Facility | Container Type and Capacity |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |

*   Attach Material Safety Sheet (MSDS), if the chemical name is commercial

**  Yearly / Daily Average

| - Procedures for Slug Control / Spill Prevention and Response |  |
| --- | --- |
|  | - Equipment |
|  | **** |
|  | - Procedures |
|  | **** |

| - Method of Disposal |  |
| --- | --- |
|  | **** |

| - Notification and Reporting Procedure |  |
| --- | --- |
|  | **** |

| - Previous Spill Events |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Description of Incident | Quantity | Date | Remedial Action |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |
|  | **** | **** | **** | **** |

| - Inspection Procedures |  |
| --- | --- |
|  | **** |

| - Training Program |  |
| --- | --- |
|  | **** |

| - Implementation Date | **** |
| --- | --- |

| - Certification |  |  |
| --- | --- | --- |
|  | I certify that the information provided in this document is to the best of·my knowledge, accurate and true, and that the incidental slug control / spill prevention measures described in this plan will be implemented as described. |  |
|  | Authorized Representative Signature | **** |
|  | Date | **** |

Combined EPA Priority Pollutants and Michigan Critical Materials List

| POLLUTANTS |  |  |
| --- | --- | --- |
| 1,1,1,2-TETRACHLOROETHANE | 1,4-DIOXACYCLOHEXANE | 2,4,5-TRICHLOROPHENOXYACETlC ACID |
| 1,1,1-TRICHLOROETHANE | 1,4-DIOXANE | 2,4,5-TRICHLOROTOLUENE |
| 1,1,2,2-TETRACHLOROETHANE | 1,5-DIAMINONAPTHALENE | 2,4,5-TRIMETHYLANILINE |
| 1,1,2,2-TETRACHLOROETHYLENE | 1,5-NAPHTHALENEDIAMINE | 2,4,5-TRIMETHYLBENZENAMINE |
| 1,1,2-TRICHLOROETHANE | 1,7-ETHYNYLESTRADIOL-3-METHYL ETHER | 2,4,6-TRICHLOROPHENOL |
| 1,1,2-TRICHLOROETHYLENE | 1-AMINO-2-METHOXY-5-METHYLBENZENE | 2,4-D |
| 1,1-DICHLOROETHANE | 1-AMINO-2-METHOXY-5-NITROBENZENE | 2,4-DIAMINOANISOLE SULFATE |
| 1,1-DICHLOROETHYLENE | 1-AMINO-2-METHYLANTHRAQUINONE | 2,4-DIAMINOTOLUENE |
| 1,1-DIMETHL-4,4'-BIPYRIDINIUM | 1-AZIRIDINE ETHANOL | 2,4-DICHLOROPHENOL |
| 1,1-OXYBIS-2-CHLOROETHANE | 1-CHLORO-2,3-EPOXYPROPANE | 2,4-DICHLOROPHENOXY ACETIC ACID |
| 1,1-THIOBIS-2-CHLOROETHANE | 1-CHLORO-2,4-DIAMINOBENZENE | 2,4-DICHLOROPHENYL-P-NITROPHENYL ETHER |
| 1,2,3,4-DIEPOXYBUTANE | 1-CHLORO-2-PROPENE | 2,4-DIMETHYLPHENOL |
| 1,2,3,4-TETRACHLOROBENZENE | 1-CHLORO-4-PHENOXYBENZENE | 2,4-DINITROPHENOL |
| 1,2,3,5-TETRACHLOROBENZENE | 1-CHLOROPROPENE | 2,4-DINITROTOLUENE |
| 1,2,3-TRICHLOROBENZENE | 1H-1,2,4-TRIAZOL-3-AMINE | 2,4-DNP |
| 1,2,4,5-TETRACHLOROBENZENE | 1-HYDROXY-2,4-DINITROBENZENE | 2,6-DIHYDROXY-5-BIS(2-CHLOROETHYL) AMINOPYRIMIDINE |
| 1,2,4-TRICHLOROBENZENE | 1-METHOXY-2-AMINO-4-NITROBENZENE | 2,6-DINITROTOLUENE |
| 1,2,5,6-DIBENZANTHRACENE | 1-METHYL-1-NITROSOUREA | 2-ACETYLAMINOFLUORENE |
| 1,2,5-TRICHLOROBENZENE | 1-METHYL-2,3-DINITROBENZENE | 2-AMINO-1-METHOXY-4-NITROBENZENE |
| 1,2,6-TRICHLOROBENZENE | 1-METHYL-2,4-DINITROBENZENE | 2-AMINO-4-CHLOROTOLUENE |
| 1,2-BENZENEDICARBOXYLIC ACID, DIOCTYL ESTER | 1-NAPHTHYL-N-METHYLCARBAMATE | 2-AMINO-4-NITROANISOLE |
| 1,2-BUTYLENE OXIDE | 1-N-BUTYL-N-NITROSOBUTANAMINE | 2-AMINOANTHRAQUINONE |
| 1,2-DIBROMO-2,2-DICHLOROETHYLDIMETHYLPHOSPHATE | 1-NITRO-2-METHYLANTHRAQUINONE | 2-AMINONAPHTHALENE |
| 1,2-DIBROMOETHANE | 2 PROPEN-1-ONE | 2-BIPHENYLOL, SODIUM SALT |
| 1,2-DICHLOROBENZENE | 2-(2-FORMYLHYDRAZINO)-4-(5-NITRO-2-FURYL) THIAZOLE | 2-CHLORO-1,3-BUTADIENE |
| 1,2-DICHLOROETHANE | 2-(CHLOROMETHYL)OXIRANE | 2-CHLOROALLYLDIETHYLDITHIOCARBAMATE |
| 1,2-DICHLOROPROPANE | 2-(P-TERT-BUTYLPHENOXY) ISOPROPYL-2-CHLOROETHYL SULFITE | 2-CHLOROBUTADIENE |
| 1,2-DIPHENYLDIAZENE | 2,2-BIOXIRANE | 2-CHLOROETHYL VINYL ETHERS |
| 1,2-DIPHENYLHYDRAZINE | 2,2-DICHLOROETYHL ETHER | 2-CHLORONAPHTHALENE |
| 1,2-EPOXYBUTANE | 2,2-DIMETHYL-4,4-METHYLENEDIANILINE | 2-CHLOROPHENOL |
| 1,2-EPOXYETHANE | 2,2-DIOXIDE-1,2-OXATHIOLANE | 2-CYANOPROPENE |
| 1,2-TRANS-DICHLOROETHYLENE | 2,3,4,5-TETRACHLOROPHENOL | 2-FAA |
| 1,3,4-TRICHLOROBENZENE | 2,3,4,6-TETRACHLOROPHENOL | 2-HYDROXY-2-METHYLPROPIONITRILE |
| 1,3-BUTADIENE | 2,3,5,6-TETRACLHOROPHENOL | 2-HYDROXYBIPHENYL SODIUM SALT |
| 1,3-DICHLOROBENZENE | 2,3,7,8-TCDD | 2-HYDROXYPROPIONITRILE |
| 1,3-DICHLOROPROPENE | 2,3,7,8-TCDF | 2-IMIDAZOLIDINETHIONE |
| 1,3-DICHORO-4-HYDROXYBENZENE | 2,3,7,8-TETRACHLORODIBENZO-P-DIOXIN | 2-METHOXY-3,4,5,6-TETRACHLOROPHENOL |
| 1,3-DIETHYLTHIOUREA | 2,3-DICHLORO-1,4-NAPHTHAQUINONE | 2-METHOXY-5-METHYLBENZENAMINE |
| 1,3-PROPANESULTONE | 2,4,5-T BUTOXYETHYL ESTER | 2-METHOXY-5-NITROANILINE |
| 1,3-PROPIOLACTONE | 2,4,5-T BUTYL ESTER | 2-METHOXY-5-NITROBENZENAMINE |
| 1,4-BENZENEDIOL | 2,4,5-TRICHLOROPHENOL | 2-METHOXYANILINE HYDROCHLORIDE |
| 1,4-DICHLOROBENZENE | 2,4,5-TRICHLOROPHENOXY ACETlC ACID BUTYL ESTER | 2-METHOXYBENZENAMINE HYDROCHLORIDE |

| POLLUTANTS |  |  |
| --- | --- | --- |
| 2-METHOXYBENZENEANAMINE | 4,4-DDE | 4-NITROSO-N-PHENYLBENZENAMINE |
| 2-METHOXYTETRACHLOROPHENOL | 4,4-DDT | 4-PYRIDINAMINE |
| 2-METHOYX-5-METHYLANILINE | 4,4'-DIAMINODIPHENYL ETHER | 4-PYRIDINE CARBOXYLIC ACID HYDAZIDE |
| 2-METHYL-1,3-DINITROBENZENE | 4,4'-DIAMINODIPHENYL SULFIDE | 5,5-DIPHENYL-2,4-IMIDAZOLIDINEDIONE |
| 2-METHYL-1-ANTHRAQUINONYLAMINE | 4,4'-METHYLENBIS(2-METHYLANILINE) | 5,5-DIPHENYL-2,4-IMIDAZOLIDINEDIONE MONOSODIUM SALT |
| 2-METHYL-1-NITRO-9,10-ANTHRACENEDIONE | 4,4'-METHYLENEBIS(N,N-DIMETHYL) BENZENAMINE | 5,5-DIPHENYLHYDANTOIN |
| 2-METHYL-1-NITROANTHRAQUINONE | 4,4'-METHYLENEBIS(O-TOLUIDINE) | 5-BIS(2-CHLOROETHYL) AMINO URACIL |
| 2-METHYL-4,6-DINITROPHENOL | 4,4'-METHYLENEBIS-2-CHLOROBENZENAMINE | 5-CHLORO-2-METHYLANILINE |
| 2-METHYL-5-CHLOROANILINE | 4,4'-METHYLENEBIS-2-METHYLBENZENAMINE | 5-CHLORO-2-METHYLBENZENAMINE |
| 2-METHYLAZIRIDINE | 4,4'-METHYLENEDI-O-TOLUIDINE | 5-CHLOR-O-TOLUIDINE |
| 2-METHYLBENZENAMINE | 4,4'-OXYBISBENZENAMINE | 5-METHYL-O-ANISIDINE |
| 2-METHYLBENZENAMINE HYDROCHLORIDE | 4,4'-OXYDIANILINE | 5-NITRO-2-METHOXYANILINE |
| 4-AMINOAZOBENZENE | 4,4'-THIOBISBENZENAMINE | 5-NITROACENAPHTHENE |
| 2-METHYLPHENOL | 4,4'-THIODIANILINE | 5-NITRO-O-ANISIDINE |
| 2-METHYLPROPENENITRILE | 4,5-BENZOPYRENE | 5-PROPYL-1,3-BENZODIOXOLE |
| 2-NAPHTHALENAMINE | 4,6-DICHLOROPHENOL | 6,7-BENZOPYRENE |
| 2-NAPHTHYLAMINE | 4,6-DINITRO-O-CRESOL | 6-METHYL-2,4-DINITROPHENOL |
| 2-NITROPHENOL | 4-AMINOAZOBENZENE | 6-METHYL-2-MERCAPTOURACIL |
| 2-NITROPROPANE | 4-AMINOBIPHENYL | 6-PROPYL-2-THIOURACIL |
| 2-OXETANONE | 4-AMINOPYRIDINE | 6-THIO-4-METHYLURACIL |
| 2-PROPENAL | 4-BROMOPHENOXYBENZENE | 9-ETHYL-9H-CARBAZOL-3-AMINE |
| 2-SEC-BUTYL-4,6-DINITROPHENOL | 4-BROMOPHENYL PHENYL ETHER | AAF |
| 3-(CHLOROMETHYL)PYRIDINE HYDROCHLORIDE | 4-CHLORO-1,2-BENZENEDIAMINE | ABIETIC ACID |
| 3-(PHENYLAZO)-2,6-PYRIDINEDIAMINE MONOHYDROCHLORIDE | 4-CHLORO-1,2-DIAMINOBENZENE | ACENAPHTHENE |
| 3,3-DICHLOROBENZIDINE | 4-CHLORO-1,3-BENZENEDIAMINE | ACENAPHTHYLENE |
| 3,3-DIMETHYL-4,4-DIAMINODIPHENYLMETHANE | 4-CHLORO-1,3-DIAMINOBENZENE | ACETOCYANOHYDRIN |
| 3,4-BENZOPYRENE | 4-CHLORODIPHENYL ETHER | ACETONE CYANOHYDRIN |
| 3,5-DIBROMO-4-HYDROXYBENZONITRILE | 4-CHLORO-M-PHENYLENEDIAMINE | ACETYLENE TETRACHLORIDE |
| 3,5-DINITRO-2-HYDROYXTOLUENE | 4-CHLOROPHENOL | ACROLEIN |
| 3-AMINO-4-METHOXYTOLUENE | 4-CHLOROPHENYL PHENYL ETHER | ACRYL ALDEHYDE |
| 3-AMINO-9-ETHYLCARBAZOLE | 4-CHLORO-PHENYLENEDIAMINE | ACRYLIC ALDEHYDE |
| 3-AMINO-9-ETHYLCARBAZOLE HYDROCHLORIDE | 4-DIMETHYLAMINO-3,5-XYLYLMETHYLCARBAMATE | ACRYLONITRILE |
| 3-CHLORO-1,2-DIBROMOPROPANE | 4-DIMETHYLAMINOAZOBENZENE | ACTI-DIONE |
| 3-CHLORO-1-PROPENE | 4-HYDROXYBUTYL BUTYL NITROSAMINE | ACTINOMYCIN C1 |
| 3-CHLOROALLYL CHLORIDE | 4-HYDROXYCHLOROBENZENE | ACTINOMYCIN D |
| 3-CHLOROPROPENYL CHLORIDE | 4-HYDROXYPHENOL | AFLATOXINS |
| 3-CHLOROPROPYLENE | 4-METHYL-1,3-BENZENEDIAMINE | ALDICARB |
| 3-METHYLPHENOL | 4-METHYL-M-PHENYLENEDIAMINE | ALDIFEN |
| 3-NITRO-6-METHOXYANILINE | 4-METHYLPHNEOL | ALDRIN |
| 3-PYRIDYLMETHYL CHLORIDE HYDROCHLORIDE | 4-METYHL-2-AMINOANISOLE | ALDRIN EPOXIDE |
| 4-(BUTYLNITROSAMINO)-1-BUTANOL | 4-NITROPHENOL | ALLYL ALDEHYDE |
| 4,4-BIPHENYLDIAMINE | 4-NITROSOMOAPHOLINE | ALLYL CHLORIDE |
| 4,4-DDD | 4-NITROSO-N-PHENYLANILINE | ALPHA-BHC |

| POLLUTANTS |  |  |
| --- | --- | --- |
| ALPHA-CHLOROTOLUENE | BAYLUSCIDE | BROMOETHYLENE |
| ALPHA-ENDOSULFAN | BENDIOCARB | BROMOFORM |
| AMETYCINE | BENLATE | BROMOMETHANE |
| AMINOBENZENE | BENOMLY | BROMOXYNIL |
| AMINOPHEN | BENZENAMINE | BUTTER YELLOW |
| AMINOTRIAZOLE | BENZENAMINE HYDROCHLORIDE | BUTYL BENZYL PHTHALATE |
| AMINOUREA HYDROCHLORIDE | BENZENE | BUTYLBUTANOLNITROSAMINE |
| AMITROLE | BENZENEAZOBENZENE | C.I. SOLVENT YELLOW 2 |
| AMlNOAZOBENZENE | BENZENOL | C.I. BASIC GREEN 4 |
| ANILAZINE | BENZIDINE (AND SALTS) | C.I. DISPERSE ORANGE 11 |
| ANILINE | BENZO(A) ANTHRACENE | C.I. SOLVENT ORANGE 35 |
| ANILINE HYDROCHLORIDE | BENZO(A) PYRENE | C.I. SOLVENT YELLOW 1 |
| ANTHRACENE | BENZO(B) FLUORANTHENE | C.I. SOLVENT YELLOW 3 |
| ANTIMONY | BENZO(B) PHENANTHRENE | C.l. 11020 |
| ANTIMONY PENTACHLORIDE | BENZO(D,E,F) CHRYSENE | CADMIUM |
| ANTIMONY PENTAFLUORIDE | BENZO(GHI) PERYLENE | CADMIUM ACETATE |
| ANTIMONY POTASSIUM TARTRATE | BENZO(K) FLUORANTHENE | CADMIUM BROMIDE |
| ANTIMONY TRIBROMIDE | BENZOEPIN | CADMIUM CHLOIRDE |
| ANTIMONY TRICHLORIDE | BENZOL | CADMIUM OXIDE |
| ANTIMONY TRIFLUORIDE | BENZYL BUTYL PHTHALATE | CADMIUM STEARATE |
| ANTIMONY TRIOXIDE | BENZYLCHLORIDE | CALCIUM HYPOCHLORITE |
| ANTIMYCIN A | BERYLLIUM | CAPTAFOL |
| ARAMITE | BERYLLIUM CHLORIDE | CAPTAN |
| AROCHLOR | BERYLLIUM FLUORIDE | CAPTAN SOW |
| ARSENIC | BERYLLIUM NITRATE | CARBAFOS |
| ARSENIC DISULFDIE | BETA-BHC | CARBAMYL HYDRAZINE |
| ARSENIC PENTOXIDE | BETA-ENDOSULFAN | CARBARYL |
| ARSENIC TRICHLORIDE | BETA-PROPIOLACTONE | CARBAZIMIDIC ACID |
| ARSENIC TRIOXIDE | BIDRIN | CARBICRON |
| ARSENIC TRISULFIDE | BIS(2-CHLOROETHOXY) METHANE | CARBOFURAN |
| ASBESTOS | BIS(2-CHLOROETHYL) ETHER | CARBON TETRACHLORIDE |
| ASBESTOS | BIS(2-CHLOROETHYL) METHYLAMINE | CARBONIC DICHLORIDE |
| AVADEX | BIS(2-CHLOROETHYL) PHOSPHORAMIDE CYCLIC PROPANOLAMIDE ESTER | CARBONYL CHLORIDE |
| AZACYCLOPROPANE | BIS(2-CHLOROETHYL) SULFIDE | CARBOPHENOTHION |
| AZINPHOS-ETHYL | BIS(2-CHLOROISOPROPYL) ETHER | CARBOPHOS |
| AZINPHOS-METHYL | BIS(2-ETHYLHEXYL)-1,2-BENZENEDICARBOXYLATE | CDEC |
| AZIRIDINE | BIS(2-ETHYLHEXYL) PHTHALATE | CELLON |
| AZOBENZENE | BIS(3-CHLORO-4-AMINOPHENYL) METHANE | CHLORAMINES |
| AZOBENZIDE | BIS(CHLOROMETHYL) ETHER | CHLORDANE |
| AZOBENZOL | BIS(DIMETHYLDITHIOCARBAMATO) ZINC | CHLORDECONE |
| AZODRIN | BIS(DIMETHYLTHIOCARBAMOYL) DISULFIDE | CHLORFENVINPHOS |
| BARBAN | BIS(TRIBUTYLTIN) OXIDE | CHLORINATED CAMPHENES |
| BASALIN | BONOFORM | CHLORINATED DIBENZOFURANS |
| BAYER 73 | BROMOCHLOPHOS | CHLORINATED DIOXINS |

| POLLUTANTS |  |  |
| --- | --- | --- |
| CHLORINE (INCLUDES HYPOCHLORITE SALTS) | CYANOGEN CHLORIDE | DIETHYLENE DIOXIDE |
| CHLOROBENZENE | CYANOGEN IODIDE | DIETHYLENE ETHER |
| CHLOROBENZILATE | CYCASIN | DIETHYLNITROSAMINE |
| CHLORODIBROMOMETHANE | CYCLOHEXATRIENE | DIETHYLSTILBESTROL |
| CHLOROETHANE | CYCLOHEXIMIDE | DIHYDROQUINONE |
| CHLOROETHYLENE | CYCLOPHOSPHAMIDE | DIHYDROSAFROLE |
| CHLOROFORM | CYGON | DIMECRON |
| CHLOROMETHANE | CYODRIN | DIMETHOATE |
| CHLOROMETHYL ETHER | CYTOXAN | DIMETHYL DISULFIDE |
| CHLOROMETHYLBENZENE | DASANIT | DIMETHYL HYDRAZINES |
| CHLOROPHENYLMETHANE | DBA | DIMETHYL MERCURY |
| CHLOROPHOS | DBCP | DIMETHYL PHTHALATE |
| CHLOROPRENE | DBP | DIMETHYL SULFATE |
| CHLOROPROPYLENE OXIDE | DDVP | DIMETHYL(2,2,2-TRICHLORO-1-HYDROXYETHYL) PHOSPHONATE |
| CHLOROTHENE | DECHLORANE | DIMETHYLBENZENE |
| CHLORPYRIFOS | DEHP | DIMETHYLENE OXIDE |
| CHROMIC ACETATE | DEHYDROABIETIC ACID | DIMETHYLENIMINE |
| CHROMIC ACID | DEKRYSIL | DIMETHYLNITROMETHANE |
| CHROMIC CHLORIDE | DELNAV | DIMETHYLNITROSAMINE |
| CHROMIC SULFATE | DELTA-BHC | DI-N-BUTYL PHTHALATE |
| CHROMIUM | DEMETON | DINITROPHENYLMETHANE |
| CHROMOUS CHLORIDE | DERTIS | DINITROTOLUENE (ALL ISOMERS) |
| CHRYSENE | DES | DINOCAP |
| CLONITRALID | DHAA | DI-N-OCTYL PHTHALATE |
| COBALT | DI(ETHYLHEXYL)PHTHALATE | DINOSEB |
| COBALT CARBONYL | DI(N-BUTYL)-1,2-BENZENEDICARBOXYLATE | DIOCTYL PHTHALATE |
| COBALTOUS BROMIDE | DI(N-BUTYL) PHTHALATE | DIOXATHION |
| COBALTOUS FORMATE | DIALLATE | DIPHENYL ETHER |
| COBALTOUS SULFAMATE | DIAZINON | DIPHENYL OXIDE |
| COPPER | DIBENZO(A,H) ANTHRACENE | DIPHENYLDIAMIDE |
| COPPER CHLORIDE | DIBROM | DIPHENYLHYDANTOIN SODIUM |
| COPPER NITRATE | DIBROMOCHLOROPROPANE | DIPHENYLNITROSAMINE |
| COPPER SULFATE | DIBUTYL PHTHALATE | DIPROPYLNITROSAMINE |
| CO-RAL | DIBUTYLNITROSAMINE | DIPTEREX |
| COSMEGEN | DICHLONE | DISULFOTON |
| COUMAPHOS | DICHLOROBROMOMETHANE | DI-SYSTON |
| COUNTER | DICHLORODIMETHYL ETHER | DITHIONE |
| CROTOTHANE | DICHLOROMETHANE | DITHIOPHOS |
| CROTOXYPHOS | DICHLORVOS | DlLANTIN |
| CUPFERRON | DICROTOPHOS | DNBP |
| CYANIDES | DIELDRIN | DNOC |
| CYANOETHYLENE | DIETHYL DISULFIDE | DNOP |
| CYANOGEN | DIETHYL MERCURY | DOP |
| CYANOGEN BROMIDE | DIETHYL PHTHALATE | DOT (INCLUDES P,P'O,P' & TECHNICAL) |

| POLLUTANTS |  |  |
| --- | --- | --- |
| DOWICIDE | FENTHION | INDENO (1,2,3-CD) PYRENE |
| DOWICIDE 1 | FICAN | ISONICOTINIC ACID HYDRAZINE |
| DOWICIDE 2 | FLAVATOXINS | ISONICOTINOYL HYDRAZINE |
| DURSBAN | FLUCHORALIN | ISONITROPROPANE |
| DYLOX | FLUORANTHENE | ISOPHORONE |
| DYRENE | FLUORENE | ISOPROPENE CYANIDE |
| EDB | FNT | ISOPROPENYL NITRILE |
| EKTAFOS | FORMALDEHYDE | ISOSAFROLEOCTYL SULFOXIDE |
| ELDOQUIN | FORMALIN | ISOSEMICARBAZIDE |
| EMBAFUME | FORMIC ALDEHYDE | KANECHLOR C |
| ENDOSULFAN | FORMOL | KARATHANE |
| ENDOSULFAN SULFATE | FUMAZONE | KEPONE |
| ENDOXAN | FURADAN | KETENE |
| ENDRIN | FURATHIOZOLE | KURON |
| ENDRIN ALDEHYDE | GAMMA-BHC | LACTONITRILE |
| EPICHLOROHDYRIN | GAMMA-HCH | LANNATE |
| EPN | GAMMA-HEXACHLOROCYCLOHEXANE | LASIOCARPINE |
| EPOXYHEPTACHLOR | GLYCIDYL CHLORIDE | LEAD |
| ETHANETHIOAMIDE | GOPHACIDE | LEAD ACETATE |
| ETHENONE | GUTHION | LEAD CHLORIDE |
| ETHENYLBENZENE | HCB | LEAD FLUORIDE |
| ETHION | HCE | LEAD NITRATE |
| ETHYL CARBAMATE | HEOD | LEAD SULFATE |
| ETHYL CHLORIDE | HEPTACHLOR | LEPTOPHOS |
| ETHYL GUTHION | HEPTACHLOR CIS-OXIDE | LINDANE |
| ETHYL MESYLATE | HEPTACHLOR EPOXIDE | LITHIUM |
| ETHYL METHANESULFONATE | HEPTACHLORONAPHTHALENE | LITHIUM CHROMATE |
| ETHYL OXIRANE | HEPZIDE | LITHIUM HYDRIDE |
| ETHYL PARATHION | HEXACHLOROBENZENE | MALACHITE GREEN |
| ETHYL THIOPYROPHOSPHATE | HEXACHLOROBUTADIENE | MALATHION |
| ETHYL URETHANE | HEXACHLOROCYCLOHEXANE (ALL ISOMERS) | MBCP |
| ETHYL-4,4-DICHLOROBENZILATE | HEXACHLOROCYCLOPENTADIENE | MBOCA |
| ETHYLBENZENE | HEXACHLOROETHANE | M-CRESOL |
| ETHYLENE CHLORIDE | HEXACHLOROETHYLENE | M-CRESYLIC ACID |
| ETHYLENE DIBROMIDE | HEXACHLORONAPTHALENE | M-DICHLOROBENZENE |
| ETHYLENE DICHLORIDE | HEXAMETHYLPHOSPHORAMIDE | MERCAPTOIMIDAZOLINE |
| ETHYLENE OXIDE | HEXAMETHYLPHOSPHORIC TRIAMIDE | MERCAPTOMETHANE |
| ETHYLENE TRICHLORIDE | HYDRAZINE | MERCAPTOPHOS |
| ETHYLENEIMINE | HYDRAZINECARBOXAMIDE | MERCAPTOTHION |
| ETHYLENETHIOUREA | HYDRAZINECARBOXAMIDE HYDROCHLORIDE | MERCURIC ACETATE |
| ETHYLETHYLENE OXIDE | HYDRAZOBENZENE | MERCURIC CHLORIDE |
| ETHYL-N-METHYLNITROSOCARBAMATE | HYDROGEN SULFIDE | MERCURIC NITRATE |
| ETO | HYDROXYBENZENE | MERCURIC SULFATE |
| FENOPROP | HYPOCHLOROUS ACID | MERCUROUS NITRATE |
| FENSULFOTHION | IMIDAN | MERCURY |

| POLLUTANTS |  |  |
| --- | --- | --- |
| MESTRANOL | MUSTARD GAS | N-NITROSODIPROPYLAMINE |
| METAPHOS | N-(2-HYDROXYETHYL) AZIRIDINE | N-NITROSO-Dl-N-PROPYLAMINE |
| METASYSTOX R | N-(2-HYDROXYETHYL) ETHYLENEIMINE | N-NITROSOMETHYLVINYLAMINE |
| METHACRYLONITRILE | N,N'-DIETHYLTHIOCARBAMIDE | N-NITROSOMORPHOLINE |
| METHALDEHYDE | N,N'-DIETHYLTHIOUREA | N-NITROSO-N-BUTYL-N(4-HYDROXYBUTYL) AMINE |
| METHANAL | N-[4-(5-NITRO-2-FURYL)-2-THIAZOLYL] ACETAMIDE | N-NITROSO-N-ETHYLUREA |
| METHANETHIOL | N-2-FLUORENYL ACETAMIDE | N-NITROSO-N-METHYLCARBAMIDE |
| METHOMYL | NALED | N-NITROSO-N-METHYLUREA |
| METHOXY-DDT | NAPHTHALENE | N-NITROSO-N-METHYLURETHANE |
| METHYL ACRYLONITRILE | NARAMYCIN A | N-NITROSO-N-PHENYLBENZENAMINE |
| METHYL ALDEHYDE | N-BUTYL-N-(4-HYDROXYBUTYL) NITROSAMINE | N-NITROSO-N-PHENYLHYDROXYLAMINE AMMONIUM SALT |
| METHYL AZOXYMETHANOL GLUCOSIDE | NEOABIETIC ACID | N-NITROSO-N-PROPYL-1-PROPANAMINE |
| METHYL BENZENE | N-ETHYL-N-(5-NITRO-2-THIAZOLYL) UREA | N-NITROSOSARCOSINE |
| METHYL BROMIDE | N-ETHYL-N-NITROSO UREA | NORQUEN |
| METHYL CHLORIDE | N-ETHYL-N-NITROSOETHANAMINE | NOTROFEN |
| METHYL CHLOROFORM | NFTA | N-PHENYL-P-NITROSOANILINE |
| METHYL DINITROBENZENE | N-HYDROXY-N-NITROSOBENZENAMINE AMMONIUM SALT | NSC 3051 |
| METHYL HYDRAZINE | NICKEL | NUDRIN |
| METHYL LACTONITRILE | NICKEL AMMONIUM SULFATE | O-AMINOANISOLE |
| METHYL MERCAPTAN | NICKEL CHLORIDE | O-AMINOAZOTOLUENE |
| METHYL NITROSOURETHANE | NICKEL HYDROXIDE | O-AMINOAZOTOLUENE |
| METHYL PARATHION | NICKEL NITRATE | O-AMINOTOLUENE |
| METHYL SULFATE | NICKEL SULFATE | O-ANISIDINE |
| METHYL THIOURACIL | NICLOSAMIDE ETHANOLAMINE SALT | O-ANISIDINE HYDROCHLORIDE |
| METHYL VINYLNITROSAMINE | NICOULINE | O-BIPHENYLOL |
| METHYL YELLOW | NIFURTHIAZOLE | O-CRESOL |
| METHYLENE CHLORIDE | NIRIDAZOLE | O-CRESYLIC ACID |
| METHYLENE DICHLORIDE | NITHIAZIDE | OCTACHLORONAPHTHALENE |
| METHYLENE OXIDE | NITROBENZENE | OCTACHLOROSTYRENE |
| METHYLENEBIS(2-CHLOROANILINE) | NITROGEN MUSTARD | OCTOIL |
| METHYLENEBIS(N,N-DIMETHYLANILINE) | NITROPENTACHLOROBENZENE | O-DICHLOROBENZENE |
| MEVINPHOS | N-METHYL FORMAMIDE | O-ETHYL-O-(4-NITROPHENYL) BENZENETHIONOPHOSPHONATE |
| MEXACARBATE | N-METHYL-2,2'-DICHLORODIETHYLAMINE | O-ETHYL-O-(4-NITROPHENYL) PHENYLPHOSPHONOTHIOATE |
| MIREX | N-METHYLBIS(2-CHLOROETHYL) AMINE | O-HYDROXYDIPHENYL |
| MITOMYCIN C | N-METHYL-N-(CARBOXMYETHYL)NITROSAMINE | OIL OF MIRBANE |
| MOCA | N-METHYL-N-NITROSOETHENAMINE | OMAL |
| MONOCHLOROBENZENE | N-METHYL-N-NITROSOGLYCINE | OMAL (NOTE T6) |
| MONOCHLOROETHYLENE | N-METKYL-N-NITROSOMETHANAMINE | O-METHOXYANILINE |
| MONOCROTALINE | N-NITROSODIETHYLAMINE | O-METHOXYPHENYLAMINE |
| MONOCROTOPHOS | N-NITROSODIMETHYLAMINE | O-METHYLANILINE |
| MONOMETHYL FORMAMIDE | N-NITROSODI-N-BUTYLAMINE | O-PHENYL PHENOL |
| MORBICID | N-NITROSODI-N-PROPYLAMINE | O-TOLUIDINE |
| MOTHOXYCHLOR | N-NITROSODIPHENYLAMINE | O-TOLUIDINE HYDROCHLORIDE |

| POLLUTANTS |  |  |
| --- | --- | --- |
| OVASTOL | PHENACHLOR | PTU |
| O-XENOL | PHENANTHRENE | PYRENE |
| OXIRANE | PHENAZOPYRIDINE HYDROCHLORIDE | QUINOL |
| OXYBISCHLOROMETHANE | PHENESTERIN | ROTENONE |
| OXYDEMETON-METHYL | PHENOBARBITOL | SELENIUM |
| P,P-BIS(DIMETHYLAMINODIPHENYL) METHANE | PHENOL | SELENIUM DIOXIDE |
| P,P'-DDD | PHENONYL | SELENIUM DISULFIDE |
| P,P'-DDE | PHENYL CHLORIDE | SELENIUM OXIDE |
| P,P-DICHLORODIPHENYLDICHLOROETHYLENE | PHENYL ETHER | SELENIUM OXYCHLORIDE |
| P-AMINOAZOBENZENE | PHENYL HYDROXIDE | SEMICARBAZIDE |
| P-AMINODIPHENYLIMIDE | PHENYLAMINE | SEMICARBAZIDE CHLORIDE |
| PARACHLOROMETA CRESOL | PHENYLETHYLENE | SEMICARBAZIDE HYDROCHLORIDE |
| PARAQUAT | PHENYLETHYLMALONYL UREA | SENDOXAN |
| PARATHION | PHENYTOIN | SEVIN |
| PARATHION-METHYL | PHENYTOIN SODIUM | SILVER |
| P-BIPHENYLAMINE | PHORATE | SILVER CHLORIDE |
| P-BROMODIPHENYL ETHER | PHOSAZETIM | SILVER NITRATE |
| PCB | PHOSDRIN | SILVEX |
| PCB-1016 (AROCHLOR 1016) | PHOSGENE | SILVEX, PROPYLENE GLYCOL BUTYL ETHER ESTER |
| PCB-1221 (AROCHLOR 1221) | PHOSMET | SODIUM FLUOROACETATE |
| PCB-1232 (AROCHLOR 1232) | PHOSPHAMIDON | SODIUM HYPOCHLORITE |
| PCB-1242 (AROCHLOR 1242) | PHOSPHORIC TRIS(DIMETHYLAMIDE) | SODIUM O-PHENYLPHENATE |
| PCB-1248 (AROCHLOR 1248) | PHOSVEL | SODIUM PENTACHLOROPHENATE |
| PCB-1254 (AROCHLOR 1254) | PHYGON | SODIUM-O-PHENYLPHENOL |
| PCB-1260 (AROCHLOR 1260) | PIPERONYL SULFOXIDE | STRYCHNINE |
| P-CHLORODIPHENYL OXIDE | P-MONOCHLOROPHENOL | STYRENE |
| P-CHLOROPHENOL | P-NITROSODIPHENYLAMINE | STYROLE |
| P-CHLOROPHENYL PHENYL ETHER | POLYBROMINATED BIPHENYLS (PBB) | SULFALLATE |
| PCNB | POLYCHLORINATED BIPHENYLS (PCB) | SULFOTEP |
| P-CRESIDINE | POLYCHLORINATED NAPHTHALENES | SULFOXIDE |
| P-CRESOL | POTASSIUM PENTACHLOROPHENATE | SULFUR HYDRIDE |
| P-CRESYLIC ACID | P-PHENOXYBROMOBENZENE | SULFUR MUSTARD |
| P-DICHLOROBENZENE | P-PHENOXYPHENYL BROMIDE | SYLVIC ACID |
| P-DIHYDROXYBENZENE | P-PHENYLAMINONITROSOBENZENE | SYSTOX |
| P-DIOXANE | P-PHENYLANILINE | TBTO |
| PENTACHLORONAPHTHALENE | PROCASIL | TCDD |
| PENTACHLORONITROBENZENE | PROCYTOX | TCDF |
| PENTACHLOROPHENOL (AND SALTS) | PROPACIL | TCP |
| PERC | PROPANOLIDE | TDE |
| PERCHLOROBENZENE | PROPENENITRILE | TEMIK |
| PERCHLOROBUTADIENE | PROPENYL CHLORIDE | TEPP |
| PERCHLOROCYCLOPENTADIENE | PROPYCIL | TERBUFOS |
| PERCHLOROETHANE | PROPYLENIMINE | TERRACLOR |
| PERCHLOROETHYLENE | PROPYLTHIOURACIL | TETRACHLOROETHYLENE |
| PERCHLOROMETHANE | PSEUDOCUMIDINE | TETRACHLOROGUAIACOL |

| POLLUTANTS |  |  |
| --- | --- | --- |
| TETRACHLOROMETHANE | TREFLAN | TUBATOXIN |
| TETRACHLORONAPHTHALENE | TRIARYL PHOSPHATE ESTERS | URACIL MUSTARD |
| TETRACHLORVINPHOS | TRIAZINE | URETHANE |
| TETRAETHYL DIPHOSPHATE | TRIBUTYLTIN (AND SALTS AND ESTERS) | VAPONA |
| TETRAETHYL DITHIOPYROPHOSHPATE | TRIBUTYLTIN ACETATE | VEGADEX |
| TETRAETHYL PYROPHOSHPATE | TRIBUTYLTIN ACRYLATE | VINYL BROMIDE |
| TETRAMETHYLDIAMINODIPHENLYMETHANE | TRIBUTYLTIN FLUORIDE | VINYL CHLORIDE |
| TETRAMETHYLTHIURAM DISULFIDE | TRIBUTYLTIN OXIDE | VINYL CYANIDE |
| TETRANITROMETHANE | TRIBUTYLTlN SULFIDE | VINYL ETHYLENE |
| THALLIUM | TRICHLORFON | VINYL TRICHLORIDE |
| THIOCARBAMIDE | TRICHLOROETHYLENE | VINYLIDENE CHLORIDE |
| THIODAN | TRICHLOROMETHANE | WHITE TAR |
| THIOLALLATE | TRICHLOROVINYLPENTACHLOROBENZENE | XYLENE (O, M AND P ISOMERS) |
| THIOPHOS | TRICRESYL PHOSPHATE | ZECTRAN |
| THIOTEPP | TRIFLURALIN | ZINC |
| THIOUREA | TRIMETHYL ORTHOPHOSPHATE | ZINC AMMONIUM CHLORIDE |
| THIRAM | TRIMETHYL PHOSPHATE | ZINC BROMIDE |
| TIMET | TRIPHENYL PHOSPHATE | ZINC CHLORIDE |
| TOK | TRIS | ZINC DIMETHYLDITHIOCARBAMATE |
| TOLUENE | TRIS (FLAME RETARDANT) | ZINC NITRATE |
| TOLUENE-2,4-DIAMINE | TRIS(2,3-DIBROMOPROPYL) PHOSPHATE | ZINC SULFATE |
| TOLYL CHLORIDE | TRITHION | ZIRAM |
| TOXAPHENE | TRIXYLENYL PHOSPHATE |  |